Introduction

The ISO organization has established a common methodology for the quantification and reporting of greenhouse gas (GHG) emissions arising from the operation of transport chains of passengers and freight. Based on this new norm ISO14083:2023, CarbonCare has amended its CO2 calculator effective July 01, 2023. An abstract of the ISO norm is published on https://www.iso.org/obp/ui/#iso:std:iso:14083:ed-1:v1:en.

Switch of standard from EN16258 to ISO14083:2023

The calculation according to ISO 14083:2023 with the new API 3.0. is available from 1.7.2023 The API 2.8. based on the standard EN16258 will still be available on the CarbonCare calculator until the end of 2023. CarbonCare will switch to ISO 14083:2023 as of 1.1.2024 for all its customers and will only support EN16258 in exceptional cases until further notice.

- BUSINESS Licensees

Our recommendation is that you keep the EN16258 standard until the end of the year 2023 and change to the new ISO14083 effective 01.01.2024. During the remaining months of 2023, testing can be done and experience with the new ISO norm and reporting can be gained.

Alternatively, you can switch to the new ISO14083 at any time after 1.7.2023 and install the new API 3.0. To apply the same standard for the whole year 2023 you can have the calculations made for all shipments from January 2023 until the time of your change, because the CO2 values are slightly different (partly new emission factors and HUB handling always included for each leg).

- BASIC, NETWORK and PLATFORM Licensees

Our recommendation is that you keep the EN16258 standard until the end of the year 2023 and change to the new ISO14083 effective 01.01.2024. During the remaining months of 2023, testing can be done and experience with the new ISO norm and reporting can be gained.

In exceptional cases we can evaluate the possible change to the new ISO14083 during the 2nd half of 2023 and change the calculation norm to ISO14083:2023.

Major changes

What is new with ISO 14083 and API 3.0 compared with the presently used EN16258.

- Calculations

- The calculation covers the entire transport chain with all “transport chain elements” (TCE) including all transshipment activities (called “HUBs”). The norm rules that only outgoing Hub activities are included.

- Figure 1 below provides an illustrative example of a freight transport chain from the point where freight leaves its last point of production or transformation (A, freight consignor) to the point where freight reaches its first non-transport related operation (B, freight consignee).

- This transport chain consists of five transport chain elements (TCEs), the GHG emissions of which are calculated separately. The first and last TCEs (TCE 1, TCE 5) represent road services (C) covering pre- and on-carriage; TCE 2 to TCE 4 represent a rail freight service (D) composed of road/rail terminal operations (TCE 2, TCE 4) and main carriage by rail transport (TCE 3). HUB handling at destination point B is not considered.

Figure 1 — Illustrative example of a multi-element freight transport chain (TCE)

B

A

| A Freight consignor B Freight consignee | C Road transport services D Rail transport service |
| --- | --- |

- CO2e values

The ISO norm requests to publish the following key figures

- TOT 	Total emissions (comparable to WTW: Well-to-Wheel)

- OPS	Operational emissions (comparable to TTW: Tank-to-Wheel)

- ENE	Energy provision (comparable to WTT: Well-to-Tank)

- EI	Emission Intensity gr/tkm or gr/t

- Various changes

- All emission factors have been updated and adapted according to ISO 14083:2023. ISO has different emission factors for North America and Europe (Rest of World) and so, the emission values might change slightly.

- “Cooling” (or Reefer) is included and available for all modes of transport (according to ISO: “Transport Chain Elements” – like ROAD, AIR, RAIL, SEA, IWW, FERRY & HUB). New is also that “Refrigerant Leaking” is included (a MUST according to ISO).

- All distances are calculated according to the “shortest feasible distance” (SFD) except AIR which still must be calculated according to “great circle distances”. Additionally, load factors are available (e.g., 0%-100%) for all ROAD transports, FERRY has the options for ROPAX and RORO ferries (small (<5000T) & large (5000-15000T), and IWW has more ships available with a more detailed classification.

Please note that the distances used on the CO2 calculator on www.carboncare.org continue to be based on the great circle distance (response times with SFD too long).

- Reporting

There will be two levels of reporting:

  - Transport level reporting

For each CO2-calculation the following values will be determined and transmitted back to the licensee.

- Concrete figures

The values of TOT, OPS, ENE and TOT EI gr/tkm (Emission Intensity in gr/tkm) plus KM and TKM will be part of the XML expressed in real figures. Furthermore, an amount for a potential compensation project calculated at the cost of CHF/EUR/USD 28/t is added for info. The detailed figures are stored by CarbonCare and can be analysed by licensees through the application “analytics & data storage”.

- Compressed additional values

Additional values with details per TCE for each shipment are compressed into a PDF which is attached to the XML.

Below an example with the layout of the PDF for the following transport/routing.

PDF

  - Organizational level reporting

The actual status is available continuously under “mycarboncare” for each licensee and access to “mycarboncare” will be available as of July 2023. Reporting with certification is made on a yearly basis. Values of licences starting during the year will be added up at the end of the first year and thereafter a yearly interval will be followed.

A CO2e-certificate will be issued at the end of each year confirming the CO2e-values having been calculated on the CarbonCare calculator during the period expressed on the certification. Please note that this certification is not tradable but merely confirmation of the values calculated. This procedure is applicable for all types of licences.

Above values of the previous years will be stored on the DB or CarbonCare and are available to the licensee.

  - Layout on “mycarboncare”

Licensees have access to their site “mycarboncare” where the values on shipment as well as on organisational level are published. A task/tool bar secures an easy browsing through the information.

Figures are constantly updated during the year and values of previous years can be accessed as well (figures in the following example are not real but shown for illustrative purpose only).

What to do now

A short overview of actions for existing customers:

- Read and digest these guidelines and the new API 3.0 together with the document “CarbonCare Emission Calculation Methodology”.

- Take a decision on when to implement API 3.0.

- Inform info@carboncare.org about the planned change schedule whenever ready

- Inform own customers of planned change from EN16258 to ISO14083:2023

- Prepare API changes (API licensees only) and implement

For any assistance and support please send a mail to support@carboncare.org (English language please) and we will do our best answer as soon as possible.

With this change to ISO14083:2023, you will be able to offer the most modern and up-to-date CO2e-values to your customers! Thank you for your trust in CarbonCare.

Kind regards

Peter Wild, Managing Partner		Peter Somaglia, Partner